REVIVE version 1.0

Author: Emil LAURENTIU

Date: Monday, 13 April 1998

INTRODUCTION

============

REVIVE is a floppy disk recovery utility.

It  also places fresh data on disk  by reformatting and rewriting all data

with the best it can recover.

Revive  is different from other existing recovery programs because special

modes of the floppy disk controller (I8272 & up) are used to restore every

possible bit of information.

The  program	does  not  use BIOS for	 disk  access.	Int  13h is entirely

rewritten for floppy and new functions like "read track", "read sector ID"

and  "seek  track"  are also included.  This  parts  are assembler. Screen

output  is  done  directly to the  video  memory.  Initial video mode (and

screen if in a text mode) are restored when program exists.

Program works well under DOS and Windows 3.x.

A  technical	explanation  of its insides  can  be  found in the "Recovery

Process" chapter.

COMMAND LINE OPTIONS

====================

Usage: REVIVE [-tsb] drive

-t:nn	number of tracks

-s:nn	number of sectors

-b	put fresh boot/fat info

The  drive must be A or B (you can omit the ':'). Every other parameter is

optional.

The geometry of the existing floppy is taken from the boot sector if there

is  a valid one. For non DOS disks or unformatted ones you can specify its

geometry using the /t & /s options.

Use /b if you want disk boot and fat(s) restored with fresh information.

RUNTIME OPTIONS

===============

At any time in the program you can press 'Esc' to exit. Note that commands

are  not executed right away. Usually	 you must wait the process executing

on the current track to end.

Pressing  'F'  at  any time switches format  mode  on or off. In this mode

tracks are only formatted and then verified. All data on current tracks is

lost.	 You may want to use this mode when you know that your disk does not

contain  any valid info after a certain track (thus speeding disk recover)

or  when you decide that a disk has far to many errors to proceed with the

reviving process.

If  format  mode is engaged right from  the beginning, boot and fat(s) are

also	rewritten.  Do not use this for	 non  DOS disks. Boot and fat(s) are

refreshed  when  you press 'Esc' or  when  the format process reaches disk

end.	In this way you can perform a "quick format" even if the disk's boot

is corrupted. Just format a few tracks (first two is enough) and then exit

this mode.

Another  mode called "Quick mode" can be switched by pressing 'Q'. In this

mode	tracks are only read and only when  an read error occurs you will be

prompted  whether  to	 proceed with track  recovery  or  not. You can skip

tracks or switch to an auto mode when no questions are asked.

At  any  time you can see the disk  map with each sector and the status of

the  last operation that was performed  on that sector. Screen is switched

to a 50 lines mode so that even 1.44Mb disk can fit in the same screen.

The  DOS  recovery  mode is not yet  finished	 in this version but you can

still walk through disk clusters and see where errors are on your disk.

The DOS recovery mode will enable you to alter files FAT chains and choose

other	 valid sectors instead of those	 corrupted. Clusters would be marked

as bad by hand or by an automated process.

RECOVERY PROCESS

================

When	a  whole  track	 read  fails  then  sectors  are  read	individually

(retries).

What	follows is engaged only if disk	 interleave is 1 and no sector shift

is present.

Track	 is read in a special mode and all symbols existing on it are stored

in  a	 16k buffer. By shifting at  bit  level and searching "sector marks"

(0xA1	 0xA1 0xA1 0xFB) some other  sectors are restored. Typically sectors

with	corrupted  ID's are restored in  this  way. Sectors are also checked

against their CRC's. These are the realigned sectors.

If  that fails each sector is read 4 times (ignoring errors) and the whole

track has 3 instances in memory thus resulting a maximum of 7 pseudo-valid

copies.

Each copy is declared valid as follows:

- sectors obtained  from whole track reads must have (after alignment) one

GAP byte (0x4E) following the CRC

- individually read sectors returned an "CRC unrecoverable" error

Each	byte from these pseudo-valid copies  is then "averaged" at bit level

(by a voting function). This ensures the closest values for each recovered

bit of information.

First	 and  last position that differ in  at least one of these copies are

then	"tried" with every possible pattern  and results are checked against

sector's CRC.

By  this process sectors with only a few bits corrupted are restored. Note

that	even when this process succeeds	 that does not necessarily mean that

sector  was restored because sector CRC is only 16 bits long and searching

in a 64k space usually gives a match.

Sectors  with	 unrecoverable	errors are  still  restored to their closest

content.

This is the best you can do without using special equipment.

BUILDING

========

Requires Watcom C and TASM

Use: "make -DSTRIP" to build

without  the "define STRIP" macro, text mode is not initialized to permit

debugging with Watcom debugger

AUTHOR

======

Emil LAURENTIU

Internet: emil@techteam.static.golden.net

HISTORY

=======

version 1.0 (13 April 1998)

- first release

STILL TO DO

===========

- dos recover mode (use the revive.dat file)

file is: (int) number of sectors with problems,

0x200 * sectors, (struct sector_s) * sectors

- Linux version ??? (ncurses for screen outpus, scroll to fit screen size,

only  bit oriented functions to be kept  in 32 bit assembler, fdd to be

rewritten in C)

LIMITATIONS

===========

- sector size is "hardcoded" to 512 bytes

- only "standard" disk sizes are supported (max 1.4M)

( 8/9/15/18 sectors per track, 40/80 tracks )

ACKNOWLEDGMENTS

===============

This program is "cardware" that means if you like this program and you use

it  you  should send me a post card.	(This is a _must_, otherwise you are

using	 my  program illegally :-). Please send me  a postcard with a nature

landscape from your country on the following address:

Emil LAURENTIU

Str. Drumea Radulescu nr.42 sect.4

Bucharest cod 75126

ROMANIA

For  questions, bug reports, suggestions, or additions to the code you can

contact me at:

Internet: emil@techteam.static.golden.net

The  latest version together with some  other articles and programs can be

found on my WEB page. The current address is:

http://techteam.static.golden.net/~emil

If that changes, use a search engine to find its new location.

My  latest email address (together with my PGP public key) can be found by

querying   the  PGP  key-servers.  Send  email  with	subject	 line  "MGET

laurentiu" to one of these servers (example: pgp-public-keys@kub.nl).